[See Diesel Fuel Transfer Document  for more Information]

DATE__/__/____        WEATHER CONDITIONS____________________________________________

Time commenced_________                       Time completed____________

|  | BRONZEWING | SHF VESSEL Name____________________ |
| --- | --- | --- |
| Vessel location |  |  |
| Responsible Person |  |  |
| Other persons |  |  |
| Litres transferred |  |  |

| 1 | Spill kit on dock close to Bronzewing |  |
| --- | --- | --- |
| 2 | Foam Fire extinguisher on dock |  |
| 3 | Communications people appointed and in place. Use radios if required |  |
| 4 | A cofferdam of oil absorbent padding is to be place around the refuelling port when the refuelling port is on deck, as well as around its fuel vents or place containers under fuel vents where possible to catch any overflow. |  |
| 5 | All the scuppers are to be fitted with absorbent padding to prevent any diesel going overboard in the event of a spill. |  |
| 6 | Have Kitty Litter on hand |  |
| 7 | Should the fuel line have a connection, the connection MUST NOT be over the water |  |
| 8 | Place NO smoking signs |  |
| 9 | Ensure there is no electrical tool use or HOT work in the immediate vicinity |  |
| 10 | Raise the Bravo code flag |  |
| 11 | Volume transferred agreed |  |
| 12 | Mobile phones switched off by all involved in fuel transfer – NOT to be used |  |
| 13 | Check that the lever valve connecting the tug fuel tank to the transfer pump is ON |  |
| 14 | Hand the fuel hose to the other vessel and place the nozzle in the fuel port |  |
| 15 | Turn ON the fuel transfer pump (circuit breaker, switch on Fuel pump) |  |
| 16 | To commence fuel flow, turn off the emergency stop button on aft engine room hatch. Operate the lever on the nozzle to start fuel flow |  |
| 17 | On completion of transfer, before removing the nozzle from the fuel port , switch OFF pump by pressing the emergency stop button then turn off the fuel pump and circuit breaker |  |
| 18 | Take the fuel hose back on board and stored in engine room |  |
| 19 | Safety equipment restowed |  |
| 20 | Fire extinguishers returned on board and in place, take down Bravo flag |  |
| 21 | Return Spill Kit to its storage place as it is shared by all at Rozelle |  |

Emergency Response

All oil / fuel  spills / discharges  - incidents - must be reported no matter what the reason.

To report an oil spill in Sydney Harbour, contact the NSW Environment Protection Authority (EPA) at 13 15 55 or Transport for NSW at 13 12 36, providing as many details as possible about the incident.

Reporting Process

- Immediate Reporting: If you witness an oil spill, it is crucial to report it as soon as possible to minimize environmental damage. Prompt reporting increases the chances of effective response and enforcement against the responsible parties.

- Contact Authorities:

2 Sources

- NSW Environment Protection Authority (EPA): Call 13 15 55 to report oil spills or pollution incidents. This line is available 24/7 and is specifically for environmental issues.

- Transport for NSW: If the pollution appears to be coming from a vessel, you can also report it by calling 13 12 36.

- Provide Detailed Information: When reporting, include as much information as possible, such as:

- The location of the spill

- The type and quantity of oil observed

- Any visible impacts on wildlife or the environment

- Photographs or video evidence, if available.

- Follow Up: After reporting, you may want to follow up with the authorities to ensure that the incident is being addressed. The EPA and other agencies will investigate the report and take necessary actions based on the severity of the incident.

By following these steps, you can help protect Sydney Harbour and its marine environment from the harmful effects of oil spills. Prompt action is essential in mitigating the impact of such incidents.

Refer to attached incident report (AMSA) also NOTICE incidents under MARPOL attached.

Master must fill in a POLREP and sent to AUS Coastal Surveillance Center, AMSA

This also targets bilge, refuelling spills, pump-out activities